Appendix C - TCP and UDP Port Assignments

Transport Control Protocol (TCP), User Datagram Protocol (UDP) ports, and Protocol Numbers are important to TCP/IP networking, intranets, and the Internet. Ports and protocol numbers provide access to a host computer. However, they also create a security hazard by allowing uninvited access. Therefore, knowing which port to allow or disable increases a network’s security. If the wrong ports or protocol numbers are disabled on a firewall, router, or proxy server as a security measure, essential services might become unavailable.

In This Appendix

Port Assignments and Protocol Numbers
Port Assignments for Commonly-Used Services
Protocol Numbers

Related Information in the Resource Kit

	For a complete listing of Well-Known Ports, Registered ports, and protocol numbers, see the Port Assignments link on the Web Resources page at http://windows.microsoft.com/windows2000/reskit/webresources.

Port Assignments and Protocol Numbers

In TCP/IP networking, a port is a mechanism that allows a computer to simultaneously support multiple communication sessions with computers and programs on the network. A port directs the request to a particular service that can be found at that IP address. The destination of a packet can be further defined by using a unique port number. The port number is determined when the connection is established.

The Internet Assigned Numbers Authority (IANA) defines the unique parameters and protocol values necessary for operation of the Internet and its future development. In the past, these numbers were documented through the RFC document series. Since that time, the assignments have been listed on the IANA Web site, constantly updated and revised when new information is available and new assignments are made. The description of the ports and protocols in this chapter is from the IANA. The Internet Engineering Task Force (IETF) is the protocol engineering and developmental arm of the Internet. Also, the Internet Society (ISOC), a professional organization of Internet experts, comments on policies and practices and oversees a number of other boards and task forces dealing with network policy issues. For more information about port assignments, see the Port Assignments link on the web resources page at http://windows.microsoft.com/windows2000/reskit/webresources.

This appendix describes Microsoft® Windows® 2000 Server and Microsoft® Windows® 2000 Professional default port assignments and IP Protocol Numbers. Protocol Numbers direct a packet to the appropriate protocol, such as UDP or TCP, that is one layer higher in the protocol stack. This appendix contains three tables:

	Table C.1 lists Well-Known Ports.

	Table C.2 lists the Registered Ports.

	Table C.3 lists TCP and UDP ports that support commonly-used Windows 2000 services.

	Table C.4 lists IP Protocol Numbers and the functions they support.

By definition, Dynamic Ports are randomly assigned and therefore cannot be known until they are assigned. Private Ports are not registered with the IANA but are used by software applications.

Port Assignments for Well-Known Ports

The Well-Known Ports are assigned by the IANA.

Ports are used in TCP or UDP communications to name the ends of logical connections that transfer data. For the purpose of providing services to unknown clients, ports were created. Table C.1 specifies the port used by the server process as its contact port. The contact port is sometimes called a Well-Known Port.

The assigned ports use a small portion of the possible port numbers. For many years the assigned ports were in the range 0-255. The range for assigned ports managed by the IANA has been expanded to the range 0-1023. The list in Table C.1 contains most of the port assignments that are significant to the Windows 2000 operating system.

Table C.1    Well-Known Ports

Port No.	Protocol	Service Name	Aliases	Comment

7	TCP	echo		Echo

7	UDP	echo		Echo

9	TCP	discard	sink null	Discard

9	UDP	discard	sink null	Discard

13	TCP	daytime		Daytime

13	UDP	daytime		Daytime

17	TCP	qotd	quote	Quote of the day

17	UDP	qotd	quote	Quote of the day

19	TCP	chargen	ttytst source	Character generator

19	UDP	chargen	ttytst source	Character generator

20	TCP	ftp-data		File Transfer

21	TCP	ftp		FTP Control

23	TCP	telnet		Telnet

25	TCP	smtp	mail	Simple Mail Transfer

37	TCP	time	 	Time

37	UDP	time	 	Time

39	UDP	rlp	resource	Resource Location Protocol

42	TCP	nameserver	name	Host Name Server

42	UDP	nameserver	name	Host Name Server

43	TCP	nicname	whois	Who Is

53	TCP	domain		Domain Name

53	UDP	domain		Domain Name Server

67	UDP	bootps	dhcps	Bootstrap Protocol Server

68	UDP	bootpc	dhcpc	Bootstrap Protocol Client

69	UDP	tftp		Trivial File Transfer

70	TCP	gopher		Gopher

79	TCP	finger		Finger

80	TCP	http	www, http	World Wide Web

88	TCP	kerberos	krb5	Kerberos

88	UDP	kerberos	krb5	Kerberos

101	TCP	hostname	hostnames	NIC Host Name Server

102	TCP	iso-tsap		ISO-TSAP Class 0

107	TCP	rtelnet		Remote Telnet Service

109	TCP	pop2	postoffice	Post Office Protocol - Version 2

110	TCP	pop3	postoffice	Post Office Protocol - Version 3

111	TCP	sunrpc	rpcbind portmap	SUN Remote Procedure Call

111	UDP	sunrpc	rpcbind portmap	SUN Remote Procedure Call

113	TCP	auth	ident tap	Authentication Sevice

117	TCP	uucp-path		UUCP Path Service

119	TCP	nntp	usenet	Network News Transfer Protocol

123	UDP	ntp	 	Network Time Protocol

135	TCP	epmap	loc-srv	DCE endpoint resolution

135	UDP	epmap	loc-srv	DCE endpoint resolution

137	TCP	netbios-ns	nbname	NETBIOS Name Service

137	UDP	netbios-ns	nbname	NETBIOS Name Service

138	UDP	netbios-dgm	nbdatagram	NETBIOS Datagram Service

139	TCP	netbios-ssn	nbsession	NETBIOS Session Service

143	TCP	imap	imap4	Internet Message Access Protocol

158	TCP	pcmail-srv	repository	PC Mail Server

161	UDP	snmp	snmp	SNMP

162	UDP	snmptrap	snmp-trap	SNMP TRAP

170	TCP	print-srv		Network PostScript

179	TCP	bgp		Border Gateway Protocol

194	TCP	irc		Internet Relay Chat Protocol

213	UDP	ipx		IPX over IP

389	TCP	ldap		Lightweight Directory Access Protocol

443	TCP	https	MCom

443	UDP	https	MCom

445	TCP			Microsoft CIFS

445	UDP			Microsoft CIFS

464	TCP	kpasswd	 	Kerberos (v5)

464	UDP	kpasswd	 	Kerberos (v5)

500	UDP	isakmp	ike	Internet Key Exchange (IPSec)

512	TCP	exec		Remote Process Execution

512	UDP	biff	comsat	Notifies users of new mail

513	TCP	login		Remote Login

513	UDP	who	whod	Database of who's logged on, average load

514	TCP	cmd	shell	Automatic Authentication

514	UDP	syslog

515	TCP	printer	spooler	Listens for incoming connections

517	UDP	talk		Establishes TCP Connection

518	UDP	ntalk

520	TCP	efs		Extended File Name Server

520	UDP	router	router routed	RIPv.1, RIPv.2

525	UDP	timed	timeserver	Timeserver

526	TCP	tempo	newdate	Newdate

530	TCP,UDP	courier	rpc	RPC

531	TCP	conference	chat	IRC Chat

532	TCP	netnews	readnews	Readnews

533	UDP	netwall		For emergency broadcasts

540	TCP	uucp	uucpd	Uucpd

543	TCP	klogin		Kerberos login

544	TCP	kshell	krcmd	Kerberos remote shell

550	UDP	new-rwho	new-who	New-who

556	TCP	remotefs	rfs rfs_server	Rfs Server

560	UDP	rmonitor	rmonitord	Rmonitor

561	UDP	monitor

636	TCP	ldaps	sldap	LDAP over TLS/SSL

749	TCP	kerberos-adm		Kerberos administration

749	UDP	kerberos-adm		Kerberos administration

Port Assignments for Registered Ports

Registered Ports, ports between 1024 and 49151, are listed by the IANA and on most systems can be used by applications or programs executed by users. Table C.2 specifies the port used by the server process as its contact port. The IANA registers uses of these ports as a convenience to the Internet community. To the extent possible, these same port assignments are used with UDP. The Registered Ports are in the numerical range of 1024-49151. The Registered Ports between 1024 and 5000 are also referred to as the Ephemeral Ports. The list below contains most of the port assignments that are significant to Windows 2000.

Table C.2    Registered Ports

Port No.	Protocol	Service Name	Aliases	Comment

1109	TCP	kpop		Kerberos POP

1167	UDP	phone		Conference calling

1433	TCP	ms-sql-s		Microsoft-SQL-Server

1433	UDP	ms-sql-s		Microsoft-SQL-Server

1434	TCP	ms-sql-m		Microsoft-SQL-Monitor

1434	UDP	ms-sql-m		Microsoft-SQL-Monitor

1512	TCP	wins		Microsoft Windows Internet Name Service

1512	UDP	wins		Microsoft Windows Internet Name Service

1524	TCP	ingreslock	ingres	Ingres

1701	UDP	l2tp		Layer Two Tunneling Protocol

1723	TCP	pptp		Point-to-point tunneling protocol

1812	UDP	radiusauth		RRAS (RADIUS authentication protocol)

1813	UDP	radacct		RRAS (RADIUS accounting protocol)

2049	UDP	nfsd	nfs	Sun NFS server

2053	TCP	knetd		Kerberos de-multiplexer

2504	UDP	nlbs		Network Load Balancing

9535	TCP	man		Remote Man Server

Port Assignments for Commonly-Used Services

There are many services associated with the Windows 2000 operating system. These services might require more than one TCP or UDP port for the service to be functional. Table C.3 shows the default ports that are used by each service mentioned.

Table C.3    Default Port Assignments for Common Services

Service Name	UDP	TCP

Browsing datagram responses of NetBIOS over TCP/IP	138

Browsing requests of NetBIOS over TCP/IP	137

Client/Server Communication		135

Common Internet File System (CIFS)	445	139, 445

Content Replication Service		560

Cybercash Administration		8001

Cybercash Coin Gateway		8002

Cybercash Credit Gateway		8000

DCOM (SCM uses udp/tcp to dynamically assign ports for DCOM)	135	135

DHCP client		67

DHCP server		68

DHCP Manager		135

DNS Administration		139

DNS client to server lookup  (varies)	53	53

Exchange Server  5.0

Client Server Communication		   135

Exchange Administrator		   135

IMAP		   143

IMAP  (SSL)		   993

LDAP		   389

LDAP  (SSL)		   636

MTA - X.400 over TCP/IP		   102

POP3		   110

POP3  (SSL)		   995

RPC		   135

SMTP		   25

NNTP		   119

NNTP  (SSL)		   563

File shares name lookup	137

File shares session		139

FTP		21

FTP-data		20

HTTP		80

HTTP-Secure Sockets Layer (SSL)		443

Internet Information Services (IIS)		80

IMAP		143

IMAP (SSL)		993

IKE (For more information, see Table C.4)	500

IPSec Authentication Header (AH) (For more information, see Table C.4)

IPSec Encapsulation Security Payload (ESP) (For more information, see Table C.4)

IRC		531

ISPMOD (SBS 2nd tier DNS registration wizard)		1234

Kerberos de-multiplexer		2053

Kerberos klogin	 	543

Kerberos kpasswd (v5)	464	464

Kerberos krb5	88	88

Kerberos kshell		544

L2TP	1701

LDAP		389

LDAP (SSL)		636

Login Sequence	137, 138	139

Macintosh, File Services  (AFP/IP)		548

Membership DPA		568

Membership MSN		569

Microsoft Chat client to server		6667

Microsoft Chat server to server		6665

Microsoft Message Queue Server 	1801	1801

Microsoft Message Queue Server 	3527	135, 2101

Microsoft Message Queue Server 		2103, 2105

MTA - X.400 over TCP/IP		102

NetBT datagrams	138

NetBT name lookups	137

NetBT service sessions		139

NetLogon	138

NetMeeting Audio Call Control		1731

NetMeeting H.323 call setup		1720

NetMeeting H.323 streaming RTP over UDP	Dynamic

NetMeeting Internet Locator Server ILS		389

NetMeeting RTP audio stream	Dynamic

NetMeeting T.120		1503

NetMeeting User Location Service		522

NetMeeting user location service  ULS		522

Network Load Balancing	2504

NNTP		119

NNTP (SSL)		563

Outlook  (see “Exchange” for ports)

Pass Through Verification	137, 138	139

POP3		110

POP3 (SSL)		995

PPTP control		1723

PPTP data (see Table C.4)

Printer sharing name lookup	137

Printer sharing session		139

Radius accounting (Routing and Remote Access)	1646 or 1813

Radius authentication (Routing and Remote Access)	1645 or 1812

Remote Install TFTP		69

RPC client fixed port session queries		1500

RPC client using a fixed port session replication		2500

RPC session ports		Dynamic

RPC user manager, service manager, port mapper		135

SCM used by DCOM 	135	135

SMTP		25

SNMP	161

SNMP Trap	162

SQL Named Pipes encryption over other protocols name lookup	137

SQL RPC encryption over other protocols name lookup	137

SQL session		139

SQL session		1433

SQL session		1024 - 5000

SQL session mapper		135

SQL TCP client name lookup	53	53

Telnet		23

Terminal Server		3389

UNIX Printing		515

WINS Manager		135

WINS NetBios over TCP/IP name service	137

WINS  Proxy	137

WINS Registration		137

WINS Replication		42

X400		102

Protocol Numbers

In an IP header, the Protocol field identifies the service in the next higher level in the protocol stack to which data is passed. Table C.4 shows these commonly used IP protocol numbers. Protocol numbers are used to configure firewalls, routers and proxies.

Table C.4    Common Protocol Numbers

Service	Protocol Number

Internet Control Message Protocol (ICMP)	1

Transmission Control Protocol (TCP)	6

User Datagram Protocol (UDP)	17

General Routing Encapsulation (PPTP data over GRE)	47

Authentication Header (AH) IPSec	51

Encapsulation Security Payload (ESP) IPSec	50

Exterior Gateway Protocol (EGP)	8

Gateway-Gateway Protocol (GGP)	3

Host Monitoring Protocol (HMP)	20

Internet Group Management Protocol (IGMP)	88

MIT Remote Virtual Disk (RVD)	66

OSPF Open Shortest Path First	89

PARC Universal Packet Protocol (PUP)	12

Reliable Datagram Protocol (RDP)	27

Reservation Protocol (RSVP) QoS	46

Additional Resources

	For a current list of protocol numbers and TCP and UDP ports, see the Internet Assigned Numbers Authority (IANA) links on the Web Resources page at http://windows.microsoft.com/windows2000/reskit/webresources.